Quiz2

SET A

### Question 1

| Location | 4655Bh | 3A314h | 53002h | 53004h |
| --- | --- | --- | --- | --- |
| Data | ABh | CDh | EFh | 00H |

Given ES = 4000h BX= A214h DS = 3000h, CS = 5234h, SS = 1234h, SI = F008h, IP = 1234h, SP = 5555h DI = AC08h.

- Determine the second top physical address of the stack segment. [Marks 2]

- MOV AX,[BX+100h], Determine the physical address of the instruction and also which value will be stored in AX after the execution.  [Marks 3]

- Determine the logical address for the memory location of which the data 00h is in. [Marks 3]

### Question 2

MOV AX, 0BCEFH

MOV BX, 5672h

ADD AX, BX

Determine the values of all the status flags, and show all the necessary calculations(such as carries). [Marks 4]

### Question 3

Write true or false beside the statements.[Marks 3]

- Physical Address calculation happens in the EU.

- EU does fetching.

- The pre-fetch queue size is 8 bytes.

Answer:

Q1.

1.Second Top offset: SP+1

P.A = SS*10+Second Top Offset

= 1234*10+(5555+1)

2.MOV AX,[BX+100h]

P.A = DS*10+BX+100

= 3000h*10+A214+100

= 30000+A214+100

= 3A314

AH, AL

AH = [3A315] = 00h

AL = [3A314] = CDh

AX = 00CDh

3.

53004h

00H

L.A = Seg No:Offset

53004h = CS:IP

53004h =  5234h: CC4h

P.A = Seg *10+offset

Offset = P.A-Seg*10

Q2.

Q3.

- Physical Address calculation happens in the EU.  False

- EU does fetching. False

- The pre-fetch queue size is 8 bytes. False

SETB

### Question 1

| Location | 3B328h | 352ACh | 53002h | 53004h |
| --- | --- | --- | --- | --- |
| Data | CFh | 4Eh | 101d | 00H |

Given ES = 3000h BX= A214h DS = 4000h, CS = 5134h, SS = 1234h, SI = F008h, IP = 1234h, SP = 5555h DI = AC08h.

- Determine the second physical address of the stack segment. [Marks 2]

- MOV AX,[DI+720h]  Determine the physical address of the instruction and also which value will be stored in AX after the execution. [Marks 3]

- Determine the logical address for the memory location of which the data EFh is in. [Marks 3]

### Question 2

MOV AX, 0FCEFh

MOV CX, 0F672h

ADD AX, CX

Determine the values of all the status flags and show all the necessary calculations(such as carries).. [Marks 4]

### Question 3

Write true or false beside the statements.[Marks 3]

- The pre-fetch queue size is 6 bits.

- IP is in BIU.

- BIU does fetching.

Answer:

Q1.

Second Offset of stack segment = FFFFh -1

P.A = SS*10+First Offset of Stack Segment

= 1234*10+(FFFF-1)

Q2.

Q3.

- The pre-fetch queue size is 6 bits. Flase

- IP is in BIU. True

- BIU does fetching. True

Quiz1

SETA

### Question 1 [CO2] [5 Points]

Suppose Mike went to the market to buy some memories for his computers. After reaching there he needed clarification because he had different sizes of address buses. a) is 32 bits, b)64 bits, in his different microprocessors.Now, according to your microprocessor knowledge, determine what sizes of memories he will buy if he wants to use them in his computers.

### Question 2 [CO1] [5 Points]

List down two key differences between them.

- Assembler and Compiler.

- RAM and ROM.

### Question 3 [CO1] [5 Points]

Determine the authenticity of the given statements, Write True/False

- We can use different sizes of memories in one microprocessor.

- Microprocessors are used to build a gaming console.

- The control bus is bidirectional.

- 8086 has an address bus of 16 bits.

- Microprocessors are used to build a sound system.

Answer:

Q1.

- 2^32 Bytes

So, 2^2 * 2^30 Bytes = 4GB

- 2^64 Bytes

Q2.

Check Slides

Q3.

- We can use different sizes of memories in one microprocessor. True

- Microprocessors are used to build a gaming console. True

- The control bus is bidirectional. False

- 8086 has an address bus of 16 bits. False

- Microprocessors are used to build a sound system. False

SETB

### Question 1 [CO2] [5 Points]

Suppose Mike went to the market to buy some memories for his computers. After reaching there you needed clarification with the different sizes of memory. a) is 8GB, b)is 64GB. Now, according to your microprocessor knowledge, determine what his Address Bus sizes should be if he wants to use those memories in his computer.

### Question 2 [CO1] [5 Points]

List down two key differences between them.

- Address Bus and Data Bus

- Microprocessor and Microcontroller.

### Question 3 [CO1] [5 Points]

Determine the authenticity of the given statements, Write True/False

- The address bus is bidirectional.

- 8086 has a data bus of 16 bits.

- Microcontrollers are used to build a washing machine.

- The control bus is bidirectional.

- We can use different sizes of memories in one microcontroller.

Answer:

Q1.

- 8 GB = 2^3 * 10^30 Bytes

So, 3+30 = 33 bits address bus needed

- 64 GB = 2^6 * 10^30 Bytes

So, 6+30 = 36 bits address bus needed

Q2.

Check Slides

Q3.

- The address bus is bidirectional. False

- 8086 has a data bus of 16 bits. True

- Microcontrollers are used to build a washing machine. True

- The control bus is bidirectional. Flase

- We can use different sizes of memories in one microcontroller. False